Veronika Hryhorieva

08/15/2026

Smart Thermostat Architecture Report

In this project, I created a prototype of a smart thermostat using a Raspberry Pi 4B. I utilized an AHT20 temperature sensor connected via I2C to monitor the room temperature. Additionally, I used GPIO for the red and blue LEDs, as well as for three buttons. One button toggles the thermostat between Off, Heat, and Cool modes, while the other two buttons adjust the temperature set point up and down. The LCD screen shows the date and time, current temperature, mode, and set point. I also implemented UART to simulate transmitting thermostat data to a server.

For the next stage of the project, I evaluated three hardware architectures: Raspberry Pi, Microchip, and NXP/Freescale. I assessed how effectively each option supports the peripherals used in the thermostat, their ability to connect to Wi-Fi and the cloud, and whether they have sufficient memory to run the program.

The Raspberry Pi 4B performed well for my prototype. It supports GPIO, I2C, and UART, enabling me to connect all the necessary components for the thermostat. A significant advantage is its built-in Wi-Fi, which simplifies connecting the thermostat to a network and eventually transmitting data to a cloud service. It also has ample RAM and uses a microSD card for storage, providing more than enough resources for this application. However, I believe the Raspberry Pi 4B offers more processing power and memory than a production thermostat truly requires (Raspberry Pi Ltd., n.d.).

The second option I explored was the Microchip PIC32MZ-W1. This architecture is designed for embedded and IoT applications. It supports GPIO, I2C, UART, and SPI, making it capable of handling the main peripherals needed for the thermostat. It also features integrated Wi-Fi, which is important because the final thermostat needs to connect to the cloud. The PIC32MZ-W1 offers up to 2 MB of Flash memory and 640 KB of RAM, which I believe is sufficient for the thermostat software while using fewer hardware resources than the Raspberry Pi. The integrated Wi-Fi also makes this architecture a good option for the next stage of the project (Microchip Technology Inc., n.d.).

The third option I evaluated was the NXP RT1060, which represents the Freescale/NXP architecture. It supports GPIO, I2C, UART, and SPI, making it compatible with the main hardware used by the thermostat. It also has sufficient performance and memory for an embedded thermostat application. However, one drawback is that Wi-Fi is not built directly into the RT1060. To connect the thermostat to Wi-Fi and the cloud, an additional wireless component would be necessary. This adds some complexity to the design compared with an architecture that already includes Wi-Fi (NXP Semiconductors, n.d.).

After reviewing the three options, I would choose the Microchip PIC32MZ-W1 for the production version of the smart thermostat. While the Raspberry Pi 4B was very useful for building and testing my prototype, it offers more resources than the final thermostat requires. The NXP option provides good performance, but it requires an additional solution for Wi-Fi connectivity. The Microchip option supports the necessary peripherals, has adequate Flash and RAM, and includes built-in Wi-Fi. Given these benefits, I believe Microchip is the best choice for meeting the thermostat requirements and providing future connectivity to the cloud.

References

Microchip Technology Inc. (n.d.). PIC32MZ W1 and WFI32 Wi-Fi microcontrollers (MCUs). Microchip Technology.

NXP Semiconductors. (n.d.). RT1060 crossover MCU with Arm Cortex-M7. NXP Semiconductors.

Raspberry Pi Ltd. (n.d.). Raspberry Pi 4 Model B product brief. Raspberry Pi Ltd.